HOSPITAL NETWORK DESIGN

with QoS & Priority Queuing

Cisco Packet Tracer — Advanced Project Guide

Computer Networking Project

Difficulty: Advanced  |  Estimated Time: 4-6 hours

# 1. Project Overview

In this project, you will design and implement a fully functional hospital network using Cisco Packet Tracer. The network must ensure that life-critical medical devices and real-time communication always receive sufficient bandwidth — even during periods of peak usage.

Quality of Service (QoS) and Priority Queuing are the core technologies used to achieve this. You will segment the hospital into VLANs, configure inter-VLAN routing, set up DHCP, implement QoS policies, and verify traffic is correctly prioritized across the network.

Project Objectives

Design a multi-VLAN hospital network with logical department segmentation

Configure inter-VLAN routing on a Layer 3 switch using SVIs

Implement DHCP for automatic IP assignment per department VLAN

Configure QoS class maps and policy maps to prioritize critical traffic

Apply Priority Queuing (PQ) and Weighted Fair Queuing (WFQ) on WAN interfaces

Verify and test that medical traffic receives highest priority under congestion

Secure the guest VLAN from accessing internal hospital systems

## 1.1 Why QoS Matters in Hospitals

Hospitals rely on a diverse mix of network applications, many of which are time-sensitive or life-critical. Without QoS, all traffic is treated equally — a large admin file download could delay a doctor's real-time ECG feed or an emergency VoIP call.

| Traffic Type | Priority Level | Consequence of Delay |
| --- | --- | --- |
| ICU patient monitoring (ECG, vitals) | CRITICAL | Patient safety risk — must never be delayed |
| VoIP emergency calls (Code Blue) | CRITICAL | Emergency response failure if dropped |
| PACS medical imaging (X-ray, MRI) | HIGH | Diagnosis delays — very bandwidth-intensive |
| Electronic Health Records (EHR) | HIGH | Clinical workflow significantly slows down |
| Video consultation (Telemedicine) | MEDIUM | Quality degrades but recovers quickly |
| Staff email and office applications | LOW | Minor inconvenience — easily buffered |
| Guest Wi-Fi (patient entertainment) | LOWEST | Can be heavily throttled without impact |

# 2. Network Topology Design

## 2.1 Department VLAN Plan

The hospital is divided into 8 VLANs — one per department plus a management VLAN for network device administration.

| Department | VLAN ID | Network Address | VLAN Name |
| --- | --- | --- | --- |
| ICU / Life Support | 10 | 192.168.10.0/24 | VLAN_ICU |
| Emergency & VoIP | 20 | 192.168.20.0/24 | VLAN_EMERGENCY |
| Radiology (PACS/Imaging) | 30 | 192.168.30.0/24 | VLAN_RADIOLOGY |
| Electronic Health Records | 40 | 192.168.40.0/24 | VLAN_EHR |
| Administration & Offices | 50 | 192.168.50.0/24 | VLAN_ADMIN |
| Telemedicine / Video | 60 | 192.168.60.0/24 | VLAN_TELEMEDICINE |
| Patient Guest Wi-Fi | 70 | 192.168.70.0/24 | VLAN_GUEST |
| Network Management | 99 | 192.168.99.0/24 | VLAN_MGMT |

## 2.2 Device Inventory

| Device | Role & Quantity |
| --- | --- |
| Cisco 3650 Multilayer Switch (Core) | 1x — Core switch, inter-VLAN routing, QoS engine |
| Cisco 2960 Access Switches | 4x — One per hospital wing, VLAN access ports |
| Cisco 2911 Router | 1x — WAN uplink, NAT, QoS on WAN interface |
| Cisco WLC + Lightweight APs | 1x WLC + 4x APs — Staff and guest wireless |
| DHCP / DNS Server | 1x — Centralized server on VLAN 40 (EHR segment) |
| ICU Patient Monitors | 4x — Simulated as PCs on VLAN 10 |
| Cisco IP Phones (VoIP) | 4x — Emergency line phones on VLAN 20 |
| Radiology Workstations | 3x — High-bandwidth machines on VLAN 30 |
| Admin PCs | 4x — Standard workstations on VLAN 50 |
| Patient Tablets (Guest) | 2x — Wi-Fi only on VLAN 70 |

## 2.3 Topology Description

- The Core Layer 3 Switch is the central hub. All access switches, the WAN router, WLC, and server connect to it.

- Four Access Switches connect via 802.1Q trunk links — one per wing (ICU, Emergency, Radiology, Admin).

- The WAN Router connects the hospital to the internet and is where WAN-side QoS policies are enforced.

- All inter-switch links carry all VLANs as 802.1Q trunk links.

Cable Types in Packet Tracer

Core Switch <--> Access Switches: GigabitEthernet straight-through cable

Core Switch <--> WAN Router: GigabitEthernet straight-through cable

Core Switch <--> WLC: GigabitEthernet straight-through cable

Access Switch <--> PCs / Phones: FastEthernet straight-through cable

WAN Router <--> Internet Cloud: Serial DCE cable (set clock rate 64000 on DCE end)

# 3. IP Addressing Scheme

## 3.1 SVI Gateway Addresses (Core Switch)

| VLAN | SVI IP Address (Default Gateway) | Subnet Mask |
| --- | --- | --- |
| VLAN 10 — ICU | 192.168.10.1 | 255.255.255.0 |
| VLAN 20 — Emergency | 192.168.20.1 | 255.255.255.0 |
| VLAN 30 — Radiology | 192.168.30.1 | 255.255.255.0 |
| VLAN 40 — EHR | 192.168.40.1 | 255.255.255.0 |
| VLAN 50 — Admin | 192.168.50.1 | 255.255.255.0 |
| VLAN 60 — Telemedicine | 192.168.60.1 | 255.255.255.0 |
| VLAN 70 — Guest | 192.168.70.1 | 255.255.255.0 |
| VLAN 99 — Management | 192.168.99.1 | 255.255.255.0 |

## 3.2 DHCP Pool Ranges

| VLAN / Department | DHCP Pool Range |
| --- | --- |
| VLAN 10 — ICU | 192.168.10.10 - 192.168.10.50 (static IPs: .10-.20 for monitors) |
| VLAN 20 — Emergency | 192.168.20.10 - 192.168.20.50 |
| VLAN 30 — Radiology | 192.168.30.10 - 192.168.30.30 |
| VLAN 40 — EHR | 192.168.40.10 - 192.168.40.100 |
| VLAN 50 — Admin | 192.168.50.10 - 192.168.50.100 |
| VLAN 60 — Telemedicine | 192.168.60.10 - 192.168.60.30 |
| VLAN 70 — Guest | 192.168.70.10 - 192.168.70.200 (large pool for patients) |

# 4. Step-by-Step Configuration

## Step 1 — Create VLANs on All Switches

! Run on Core Switch AND all 4 Access Switches

enable

configure terminal

vlan 10

name VLAN_ICU

vlan 20

name VLAN_EMERGENCY

vlan 30

name VLAN_RADIOLOGY

vlan 40

name VLAN_EHR

vlan 50

name VLAN_ADMIN

vlan 60

name VLAN_TELEMEDICINE

vlan 70

name VLAN_GUEST

vlan 99

name VLAN_MGMT

exit

## Step 2 — Configure Trunk Links (Core Switch)

! Configure all uplinks to access switches as 802.1Q trunks

interface GigabitEthernet1/0/1

switchport mode trunk

switchport trunk encapsulation dot1q

switchport trunk allowed vlan 10,20,30,40,50,60,70,99

description Trunk_to_AccessSW1_ICU_Wing

!

! Repeat for Gi1/0/2, Gi1/0/3, Gi1/0/4 (other wings)

interface GigabitEthernet1/0/2

switchport mode trunk

switchport trunk encapsulation dot1q

switchport trunk allowed vlan 10,20,30,40,50,60,70,99

description Trunk_to_AccessSW2_Emergency_Wing

## Step 3 — Configure Access Ports on Access Switches

! ICU Wing Switch — assign ports to VLANs

interface range FastEthernet0/1 - 4

switchport mode access

switchport access vlan 10

description ICU_Patient_Monitor

spanning-tree portfast

!

! Voice VLAN for IP Phones (data on VLAN 50, voice on VLAN 20)

interface range FastEthernet0/5 - 8

switchport mode access

switchport access vlan 50

switchport voice vlan 20

description VoIP_Phone_Port

spanning-tree portfast

## Step 4 — Configure SVIs for Inter-VLAN Routing (Core Switch)

! Enable Layer 3 routing on the multilayer switch

ip routing

! Create SVI for VLAN 10 (ICU) — acts as default gateway

interface Vlan10

ip address 192.168.10.1 255.255.255.0

ip helper-address 192.168.40.10

description Gateway_ICU

no shutdown

!

interface Vlan20

ip address 192.168.20.1 255.255.255.0

ip helper-address 192.168.40.10

description Gateway_Emergency

no shutdown

!

interface Vlan30

ip address 192.168.30.1 255.255.255.0

ip helper-address 192.168.40.10

no shutdown

!

! Repeat same pattern for VLAN 40, 50, 60, 70, 99

interface Vlan40

ip address 192.168.40.1 255.255.255.0

description Gateway_EHR_Server

no shutdown

## Step 5 — Configure DHCP on the Server

In Packet Tracer, click the DHCP Server > Desktop > DHCP. Create one pool per VLAN:

| Field | Example — VLAN 10 (ICU Pool) |
| --- | --- |
| Pool Name | ICU_POOL |
| Default Gateway | 192.168.10.1 |
| DNS Server | 192.168.40.10 |
| Start IP Address | 192.168.10.10 |
| Subnet Mask | 255.255.255.0 |
| Max Users | 50 |

Repeat for all 7 department VLANs. The ip helper-address on each SVI forwards DHCP broadcast requests to the server at 192.168.40.10.

# 5. QoS & Priority Queuing Configuration

## 5.1 QoS Strategy — MQC Framework

Cisco IOS uses the Modular QoS CLI (MQC) which has three steps: classify traffic with class-map, define actions with policy-map, then apply the policy to an interface with service-policy.

| MQC Step | Purpose | Command |
| --- | --- | --- |
| class-map | Identify and match specific traffic types | class-map match-any NAME |
| policy-map | Define what to do with each traffic class | policy-map NAME |
| service-policy | Apply the policy in/out on a router interface | service-policy output NAME |

## 5.2 DSCP Marking Plan

Each traffic class is marked with a DSCP value. Routers use these markings hop-by-hop to make consistent queuing decisions across the entire network.

| Traffic Class | DSCP Value | Name | Queuing Behavior |
| --- | --- | --- | --- |
| ICU Monitoring + VoIP | 46 | EF (Expedited Forwarding) | Strict Priority — served first always |
| Medical Imaging (PACS) | 34 | AF41 (Assured Forward) | 25% guaranteed bandwidth |
| EHR / Clinical Systems | 26 | AF31 (Assured Forward) | 20% guaranteed bandwidth |
| Telemedicine Video | 18 | AF21 (Assured Forward) | 10% guaranteed bandwidth |
| Admin / Office | 10 | AF11 (Assured Forward) | 10% guaranteed bandwidth |
| Guest Wi-Fi | 0 | BE (Best Effort) | Leftovers only + policed at 2 Mbps |

## 5.3 Class Maps

! ---- TRAFFIC CLASSIFICATION (class-maps) ----

! Class 1: Critical — ICU Monitoring + VoIP Emergency

class-map match-any CM_CRITICAL

match dscp ef

match access-group name ACL_ICU_VLAN

description ICU_Monitors_and_VoIP_Emergency

!

! Class 2: High — Medical Imaging PACS

class-map match-any CM_PACS_IMAGING

match dscp af41

match access-group name ACL_RADIOLOGY

description Radiology_PACS_MRI_Xray

!

! Class 3: High — EHR Systems

class-map match-any CM_EHR

match dscp af31

match access-group name ACL_EHR

description Electronic_Health_Records

!

! Class 4: Medium — Telemedicine Video

class-map match-any CM_TELEMEDICINE

match dscp af21

description Telemedicine_Video_Conferencing

!

! Class 5: Low — Admin / Office

class-map match-any CM_ADMIN

match dscp af11

description Admin_Email_Office_Apps

!

! Class 6: Lowest — Guest Wi-Fi (Best Effort)

class-map match-any CM_GUEST

match dscp default

match access-group name ACL_GUEST_VLAN

description Patient_Guest_WiFi

## 5.4 ACLs for Traffic Matching

! ACLs to match traffic from specific VLANs

ip access-list extended ACL_ICU_VLAN

permit ip 192.168.10.0 0.0.0.255 any

!

ip access-list extended ACL_RADIOLOGY

permit ip 192.168.30.0 0.0.0.255 any

!

ip access-list extended ACL_EHR

permit ip 192.168.40.0 0.0.0.255 any

!

ip access-list extended ACL_GUEST_VLAN

permit ip 192.168.70.0 0.0.0.255 any

## 5.5 Policy Map — Priority Queuing

! ---- POLICY MAP with Priority Queuing ----

policy-map PM_HOSPITAL_QOS

!

! CRITICAL: Strict Priority Queue — gets bandwidth first, always

class CM_CRITICAL

priority percent 30

set dscp ef

!

! HIGH: PACS Imaging — guaranteed 25% bandwidth

class CM_PACS_IMAGING

bandwidth percent 25

set dscp af41

queue-limit 64 packets

!

! HIGH: EHR — guaranteed 20% bandwidth

class CM_EHR

bandwidth percent 20

set dscp af31

!

! MEDIUM: Telemedicine — guaranteed 10%

class CM_TELEMEDICINE

bandwidth percent 10

set dscp af21

!

! LOW: Admin — guaranteed 10%

class CM_ADMIN

bandwidth percent 10

set dscp af11

!

! LOWEST: Guest — only 5% + policed at 2 Mbps

class CM_GUEST

bandwidth percent 5

police rate 2000000 bps

conform-action transmit

exceed-action drop

!

! All remaining unclassified traffic — fair-queue

class class-default

fair-queue

set dscp default

Understanding Priority vs Bandwidth

'priority percent 30' = Strict Priority Queue: Critical traffic ALWAYS served first.

ICU/VoIP packets jump to the front of every other queue — zero delay guaranteed.

'bandwidth percent X' = WFQ: Remaining bandwidth (after PQ is satisfied) is shared

proportionally between the other classes based on their percentages.

Bandwidth math: 30% PQ + 25% + 20% + 10% + 10% + 5% = 100% -- fully allocated.

## 5.6 Apply Policy to WAN Interface

! Apply outbound QoS policy on the WAN serial interface (WAN Router)

interface Serial0/0/0

description WAN_Link_to_Internet

ip address 10.0.0.1 255.255.255.252

service-policy output PM_HOSPITAL_QOS

bandwidth 10000

no shutdown

# 6. NAT Configuration

Configure NAT overload (PAT) on the WAN router so all hospital VLANs can access the internet via one public IP:

interface Serial0/0/0

ip nat outside

!

interface GigabitEthernet0/0

ip nat inside

!

! Permit all internal subnets for NAT translation

ip access-list standard ACL_NAT_INSIDE

permit 192.168.10.0 0.0.0.255

permit 192.168.20.0 0.0.0.255

permit 192.168.30.0 0.0.0.255

permit 192.168.40.0 0.0.0.255

permit 192.168.50.0 0.0.0.255

permit 192.168.60.0 0.0.0.255

permit 192.168.70.0 0.0.0.255

!

ip nat inside source list ACL_NAT_INSIDE interface Serial0/0/0 overload

ip route 0.0.0.0 0.0.0.0 10.0.0.254

# 7. Verification & Testing

## 7.1 VLAN & Routing Verification

| Command | What to Verify |
| --- | --- |
| show vlan brief | All 8 VLANs are Active |
| show interfaces trunk | All uplinks trunking with correct VLANs allowed |
| show ip interface brief | All SVI interfaces are up/up with correct IPs |
| show ip route | Connected routes for all 192.168.x.0/24 networks |
| ping 192.168.20.1 (from VLAN10) | Inter-VLAN routing works between departments |
| show ip dhcp binding | Devices have received IPs from DHCP |
| show ip nat translations | NAT entries appear when internet traffic flows |

## 7.2 QoS Verification

! Show QoS policy applied to the WAN interface

show policy-map interface Serial0/0/0

! View all class-map definitions

show class-map

! Detailed statistics — packet counts per class

show policy-map interface Serial0/0/0 output

Look for: CM_CRITICAL shows packets matched and queued in priority queue. CM_GUEST shows policing drops when exceeding 2 Mbps. All other classes show non-zero byte counts.

## 7.3 Congestion Test

- Generate sustained bulk traffic from Admin PCs (VLAN 50) to simulate network congestion.

- Simultaneously send ICMP pings from an ICU monitor (VLAN 10) to an external address.

- Observe that ICU ping replies continue with consistent low latency.

- Check show policy-map output — CM_ADMIN queue depth grows while CM_CRITICAL stays drained.

- Stop admin traffic — verify ICU traffic was unaffected throughout the test.

Expected QoS Test Results

ICU / VoIP traffic (DSCP EF): queue drops = 0, all packets transmitted on time

Admin traffic (DSCP AF11): queue drops > 0 during congestion [CORRECT behavior]

Guest traffic (DSCP BE): policing drops appear above 2 Mbps [CORRECT behavior]

This proves Priority Queuing works: critical medical traffic is protected at all times.

# 8. Security Hardening

## 8.1 Block Guest from Internal VLANs

! Apply inbound ACL on VLAN 70 SVI to block guest access to internal subnets

ip access-list extended ACL_BLOCK_GUEST

deny   ip 192.168.70.0 0.0.0.255 192.168.10.0 0.0.0.255

deny   ip 192.168.70.0 0.0.0.255 192.168.20.0 0.0.0.255

deny   ip 192.168.70.0 0.0.0.255 192.168.30.0 0.0.0.255

deny   ip 192.168.70.0 0.0.0.255 192.168.40.0 0.0.0.255

deny   ip 192.168.70.0 0.0.0.255 192.168.50.0 0.0.0.255

deny   ip 192.168.70.0 0.0.0.255 192.168.60.0 0.0.0.255

permit ip any any

!

interface Vlan70

ip access-group ACL_BLOCK_GUEST in

## 8.2 SSH & Device Hardening

- Enable SSH v2 on all devices — disable Telnet on all VTY lines

- Assign all unused access ports to a dead VLAN (e.g., VLAN 999) and shut them down

- Use VLAN 99 exclusively for switch management — never route VLAN 99 externally

! Enable SSH, disable Telnet

ip domain-name hospital.local

crypto key generate rsa modulus 2048

ip ssh version 2

line vty 0 4

transport input ssh

login local

# 9. Project Summary & Deliverables

## 9.1 What You Built

- 8-VLAN hospital network covering all major clinical and administrative departments

- Inter-VLAN routing via Layer 3 Core Switch with per-VLAN SVI interfaces

- Centralized DHCP server with ip helper-address relay across all VLANs

- 6-class QoS policy using MQC: Priority Queue for ICU/VoIP, WFQ for all others

- DSCP marking for consistent hop-by-hop QoS treatment across the network

- Traffic policing on Guest VLAN capped at 2 Mbps

- NAT overload (PAT) for full internet access from all internal VLANs

- ACL blocking guest VLAN from accessing any internal hospital system

- SSH v2 hardening on all network infrastructure devices

## 9.2 Technologies Practiced

| Technology | Skills Demonstrated |
| --- | --- |
| VLAN Design | Real-world logical segmentation of an enterprise network |
| SVI / Inter-VLAN | Layer 3 switch configuration and IP routing between VLANs |
| DHCP Relay | ip helper-address with centralized server across multiple VLANs |
| MQC QoS | class-map, policy-map, service-policy full workflow |
| Priority Queuing | Strict PQ for life-critical traffic under congestion |
| DSCP Marking | End-to-end QoS signaling using EF, AF, and BE markings |
| Traffic Policing | Rate-limiting guest bandwidth with police command |
| NAT / PAT | Internet access for private RFC1918 address space |
| Extended ACLs | Named ACLs for VLAN isolation and traffic filtering |
| SSH Hardening | Secure management plane configuration |

## 9.3 Final Submission Checklist

Checklist Before Submission

[ ] Packet Tracer .pkt file with complete topology saved

[ ] All VLANs active: show vlan brief

[ ] All SVIs up/up: show ip interface brief

[ ] DHCP working: at least 1 device per VLAN has received an IP

[ ] Inter-VLAN routing: ICU VLAN can ping Admin VLAN gateway

[ ] QoS policy applied: show policy-map interface Serial0/0/0

[ ] Guest blocked internally: ping from VLAN 70 to VLAN 10 fails

[ ] NAT working: internal device can reach simulated internet

[ ] SSH configured: Telnet blocked on all VTY lines

[ ] Written report: topology diagram, IP scheme, QoS policy explanation

Project Complete — Outstanding Work!

This project demonstrates enterprise healthcare network design with industry-standard QoS — a rare skill at student level.